Proposal

Kirkwood Heating & Air Conditioning

P.O. Box 220405

Kirkwood, MO  63122-9998

(314) 968-3453 FAX (314) 968-3412

Proposal Submitted To	Phone	Date

Debbie Akins                           314-702-3214                           9/20/2022

637 Fieldcrest                          snikbbed@gmail.com

Crestwood 63126

We hereby submit specifications and estimates for:

3.5 Ton, 20 SEER System Option:

Install Mitsubishi multi-zone minisplit complete with 4 indoor heads.  Install 3.5 ton MXZ5C42NAHZU1 Hyper Heat 20 SEER outdoor unit with matching MSZFS indoor high wall air handlers.  Install 18,000 btuh air handler in the kitchen over the cabinets.  Install 9,000 btuh air handlers in each bedroom.  The bathroom and mechanical/laundry room will not be directly conditioned but should stay at a similar temperature as the rooms they’re near.  Install PACMKA52BC branch box in the laundry room as a common point for the air handlers.  Install a 3/8 x 5/8 lineset from the branch box to the outdoor unit as well as 240 volt power and 16/2 shielded communicating cable.  Install ¼ x 3/8 lineset to each bedroom unit from the branch box and ¼ x ½ lineset to the kitchen/living room unit.

The outdoor unit will be relocated further down the exterior wall where a concrete pad currently sits.  Level a pad and set the new unit on heat pump risers.  Pull new circuit to the unit as necessary as the new A/C requires a 45 amp circuit and the existing is a 20 amp circuit.

Use the lowered ceiling area of the hallway as the chase for refrigerant lines, electric, and drains.  The drains may be terminated in the mechanical room drain and do not necessarily need to be run all the way to the exterior.  The ductwork or the open attic/soffit space may be used as the chase depending on the height needed for each indoor unit to attain constant slope for a gravity drain.

Includes 10 year parts warranty and 3 year labor warranty.  Includes complete installation and clean-up.  Homeowner will demo and dispose of the indoor equipment before the installation.

Price- $17,258

- $900 Ameren Rebate

= $16,358 due upon completion

$550 Cash/Check discount available to avoid credit card or financing fees

or

18 mo/0% financing available at $909/mo

Longer financing options available upon request

*Qualifies for $300 federal tax credit which may increase with new Inflation Relief Bill

This system has the capability to add a 5th zone for the bathroom if desired.  Add additional 6,000 btuh air handler in the bathroom for $1,610

All equipment is readily available except the branch box which is expected to be available sometime in October.  May be able to be sourced alternately from another supplier more quickly

3 Ton, 23 SEER System Option:

Install MXZ-SM36NAMHZ-U1 3 ton outdoor unit with matching MSZFS indoor units.  Install 9k btuh units in each bedroom and 15k btuh unit in the kitchen/living room

Install everything else in the same fashion as above

Price- $16,003

- $900 Ameren Rebate

= $15,103 due upon completion

$550 Cash/Check discount available to avoid credit card or financing fees

or

18 mo/0% financing available = $839/mo

Longer financing options available upon request

*Qualifies for $300 federal tax credit which may increase with new Inflation Relief Bill

Outdoor unit is not available and there is no known delivery date due to backorder.  This 3 ton system is more similarly sized to what you currently have but I quoted the 3.5 ton due to available inventory

Authorized Signature                         Tyler Menos

Note:  This proposal may be withdrawn if not accepted within 30 days.

All material is guaranteed to be as specified.  All work to be completed in a workmanlike manner according to standard practices.  Any alteration or deviation from above specifications involving extra costs will be executed only upon written orders, and will become an extra charge over and above the estimate.  All agreements contingent upon strikes, accidents, or delays beyond our control.  Owner to carry fire, tornado, and other necessary insurances. Our workers are fully covered by Workman’s Compensation Insurance

Acceptable of Proposal – The above prices, specifications and conditions are satisfactory and are hereby accepted.  You are authorized to do the work as specified above.

Date of Acceptance                                                          Signature___________________________________